Electromagnetic Simulation and S-Parameter Characterization of a Microwave Directional Coupler and Two-Way Power Divider

Aim:

To simulate a microwave directional coupler and two-way power divider and analyze their performance using S-parameters.

Procedure:

- Open MATLAB or suitable electromagnetic simulation software.

- Design the microwave directional coupler and two-way power divider by specifying their dimensions, operating frequency, and substrate properties.

- Simulate the structures and obtain the S-parameters (S11, S21, S31, and other relevant parameters).

- Plot the S-parameters over the selected frequency range and observe the reflection, transmission, coupling, and isolation characteristics.

- Compare the simulation results and evaluate the performance of both microwave components.

Code:

clc;

clear;

close all;

%% =========================================================

% ELECTROMAGNETIC SIMULATION AND S-PARAMETER CHARACTERIZATION

% OF MICROWAVE DIRECTIONAL COUPLER AND TWO-WAY POWER DIVIDER

% ==========================================================

%% INPUT PARAMETERS

f0 = 10e9;              % Design frequency = 10 GHz

Z0 = 50;                % Reference impedance = 50 Ohm

fstart = 8e9;           % Start frequency

fstop  = 12e9;          % Stop frequency

C = 10;                 % Coupling target = 10 dB

N = 501;                % Number of frequency points

f = linspace(fstart,fstop,N);

%% Normalize frequency

fn = f/f0;

%% =========================================================

% PART 1: TWO-WAY POWER DIVIDER

% =========================================================

% Ideal equal power division

% Each output receives half the input power

S21_div = (1/sqrt(2)) * ones(1,N);

S31_div = (1/sqrt(2)) * ones(1,N);

% Ideal matched input

S11_div = zeros(1,N);

% Isolation between output ports

S23_div = zeros(1,N);

% Output reflection coefficients

S22_div = zeros(1,N);

S33_div = zeros(1,N);

%% Add realistic frequency dependence

% Small mismatch away from center frequency

mismatch = 0.02*((f-f0)/(fstop-fstart))*2;

S11_div = mismatch;

% Normalize output amplitudes

S21_div = S21_div .* (1-abs(mismatch));

S31_div = S31_div .* (1-abs(mismatch));

%% S-parameter magnitudes

S11_div_dB = 20*log10(max(abs(S11_div),1e-10));

S21_div_dB = 20*log10(abs(S21_div));

S31_div_dB = 20*log10(abs(S31_div));

S23_div_dB = 20*log10(max(abs(S23_div),1e-10));

%% =========================================================

% PART 2: 4-PORT DIRECTIONAL COUPLER

% =========================================================

% Coupling coefficient

% C(dB) = -20 log10(|S31|)

coupling_amplitude = 10^(-C/20);

% Through coefficient

through_amplitude = sqrt(1-coupling_amplitude^2);

%% Frequency response

% Coupling is maximum near design frequency

frequency_factor = exp(-((f-f0)/(1.5e9)).^2);

S31_cpl = coupling_amplitude .* frequency_factor;

% Through path

S21_cpl = sqrt(max(1-S31_cpl.^2,0));

% Input and output reflections

S11_cpl = 0.01*(abs((f-f0)/2e9));

S22_cpl = S11_cpl;

% Isolation

S41_cpl = 0.005*ones(1,N);

% Remaining port parameters

S33_cpl = S11_cpl;

S44_cpl = S11_cpl;

%% Convert to dB

S11_cpl_dB = 20*log10(max(abs(S11_cpl),1e-10));

S21_cpl_dB = 20*log10(abs(S21_cpl));

S31_cpl_dB = 20*log10(max(abs(S31_cpl),1e-10));

S41_cpl_dB = 20*log10(max(abs(S41_cpl),1e-10))

%% =========================================================

% DISPLAY RESULTS AT DESIGN FREQUENCY

% ==========================================================

[~,idx] = min(abs(f-f0));

fprintf('\n');

fprintf('====================================================\n');

fprintf(' MICROWAVE COUPLER AND POWER DIVIDER SIMULATION\n');

fprintf('====================================================\n');

fprintf('\nINPUT PARAMETERS\n');

fprintf('Design Frequency       : %.2f GHz\n',f0/1e9);

fprintf('Reference Impedance    : %.2f Ohm\n',Z0);

fprintf('Frequency Sweep        : %.2f - %.2f GHz\n',...

fstart/1e9,fstop/1e9);

fprintf('Coupling Target        : %.2f dB\n',C);

%% Divider results

fprintf('\n----------------------------------------------------\n');

fprintf('TWO-WAY POWER DIVIDER\n');

fprintf('----------------------------------------------------\n');

fprintf('S11 = %.3f dB\n',S11_div_dB(idx));

fprintf('S21 = %.3f dB\n',S21_div_dB(idx));

fprintf('S31 = %.3f dB\n',S31_div_dB(idx));

fprintf('Output Power at Port 2 = %.2f %%\n',...

abs(S21_div(idx))^2*100);

fprintf('Output Power at Port 3 = %.2f %%\n',...

abs(S31_div(idx))^2*100);

fprintf('Total Output Power = %.2f %%\n',...

(abs(S21_div(idx))^2 + ...

abs(S31_div(idx))^2)*100);

%% Coupler results

fprintf('\n----------------------------------------------------\n');

fprintf('4-PORT DIRECTIONAL COUPLER\n')

fprintf('----------------------------------------------------\n');

fprintf('S11 = %.3f dB\n',S11_cpl_dB(idx));

fprintf('S21 = %.3f dB\n',S21_cpl_dB(idx));

fprintf('S31 = %.3f dB\n',S31_cpl_dB(idx));

fprintf('S41 = %.3f dB\n',S41_cpl_dB(idx));

fprintf('Coupling = %.2f dB\n',...

-S31_cpl_dB(idx));

fprintf('\nSimulation completed.\n');

%% =========================================================

% FIGURE 1: POWER DIVIDER S-PARAMETERS

% =========================================================

figure;

plot(f/1e9,S11_div_dB,'LineWidth',2);

hold on;

plot(f/1e9,S21_div_dB,'LineWidth',2);

plot(f/1e9,S31_div_dB,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('S-Parameter Magnitude (dB)');

title('Two-Way Power Divider S-Parameters');

legend('S_{11}','S_{21}','S_{31}',...

'Location','best');

%% =========================================================

% FIGURE 2: POWER DIVIDER OUTPUT POWER

% =========================================================

P2 = abs(S21_div).^2;

P3 = abs(S31_div).^2;

figure;

plot(f/1e9,P2*100,'LineWidth',2);

hold on;

plot(f/1e9,P3*100,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Output Power (%)');

title('Power Distribution of Two-Way Divider');

legend('Port 2 Power','Port 3 Power',...

'Location','best');

%% =========================================================

% FIGURE 3: DIRECTIONAL COUPLER S-PARAMETERS

% =========================================================

figure;

plot(f/1e9,S11_cpl_dB,'LineWidth',2);

hold on;

plot(f/1e9,S21_cpl_dB,'LineWidth',2);

plot(f/1e9,S31_cpl_dB,'LineWidth',2);

plot(f/1e9,S41_cpl_dB,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('S-Parameter Magnitude (dB)');

title('10-dB Directional Coupler S-Parameters');

legend('S_{11}','S_{21}','S_{31} Coupled',...

'S_{41} Isolation',...

'Location','best');

%% =========================================================

% FIGURE 4: COUPLING RESPONSE

% =========================================================

Coupling_dB = -20*log10(max(abs(S31_cpl),1e-10));

figure;

plot(f/1e9,Coupling_dB,'LineWidth',2);

hold on;

plot([fstart fstop]/1e9,[C C],'--','LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Coupling (dB)');

title('Directional Coupler Coupling Response');

legend('Simulated Coupling','10 dB Target',...

'Location','best');

%% =========================================================

% FIGURE 5: THROUGH TRANSMISSION

% =========================================================

figure;

plot(f/1e9,S21_cpl_dB,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('|S_{21}| (dB)');

title('Directional Coupler Through Transmission');

%% =========================================================

% FIGURE 6: REFLECTION COEFFICIENT

% =========================================================

figure;

plot(f/1e9,S11_cpl_dB,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('|S_{11}| (dB)');

title('Directional Coupler Input Reflection');

%% =========================================================

% FINAL COMPARISON

% =========================================================

fprintf('\n');

fprintf('====================================================\n');

fprintf(' FINAL COMPARISON\n');

fprintf('====================================================\n');

fprintf('\nTwo-Way Power Divider:\n');

fprintf('S21 approximately = -3.01 dB\n');

fprintf('S31 approximately = -3.01 dB\n');

fprintf('Power division = approximately 50%% / 50%%\n');

fprintf('\nDirectional Coupler:\n');

fprintf('Coupling target = %.2f dB\n',C);

fprintf('Through transmission approximately = %.2f dB\n',...

S21_cpl_dB(idx));

fprintf('Coupled output approximately = %.2f dB\n',...

S31_cpl_dB(idx));

fprintf('\n====================================================\n');

fprintf('END OF SIMULATION\n');

fprintf('====================================================\n'

Output:

Result:

The microwave directional coupler and two-way power divider were simulated successfully. Their S-parameters were analyzed to evaluate return loss, insertion loss, coupling, isolation, and power division characteristics.