Simulation-Based Investigation of Microwave Semiconductor Devices and Frequency-Conversion Characteristics of a Microwave Mixer

Aim:

To simulate microwave semiconductor devices and analyze the frequency-conversion characteristics of a microwave mixer.

Procedure:

- Open MATLAB or suitable microwave simulation software.

- Select a microwave semiconductor device, such as a Schottky diode, and define its operating parameters.

- Design a microwave mixer by applying the RF signal and local oscillator (LO) signal.

- Simulate the mixer and observe the output intermediate frequency (IF) signal.

- Analyze the input and output frequencies, conversion loss, and frequency-conversion characteristics.

Code:

clc;

clear;

close all;

%% =========================================================

% MICROWAVE MIXER FREQUENCY-CONVERSION SIMULATION

% =========================================================

%% INPUT PARAMETERS

fRF = 10e9;             % RF frequency = 10 GHz

fLO = 9.5e9;            % LO frequency = 9.5 GHz

PRF_dBm = -10;          % RF input power = -10 dBm

PLO_dBm = 7;            % LO power = +7 dBm

ZIF = 50;               % IF load impedance = 50 Ohm

%% =========================================================

% FREQUENCY CONVERSION

% ==========================================================

% Difference frequency

fIF = abs(fRF - fLO);

% Sum frequency

fSUM = fRF + fLO;

fprintf('\n');

fprintf('====================================================\n');

fprintf(' MICROWAVE MIXER FREQUENCY-CONVERSION SIMULATION\n');

fprintf('====================================================\n');

fprintf('\nINPUT PARAMETERS\n');

fprintf('RF Frequency       = %.2f GHz\n',fRF/1e9);

fprintf('LO Frequency       = %.2f GHz\n',fLO/1e9);

fprintf('RF Input Power     = %.2f dBm\n',PRF_dBm);

fprintf('LO Power           = %.2f dBm\n',PLO_dBm);

fprintf('IF Load            = %.2f Ohm\n',ZIF);

fprintf('\nFREQUENCY CONVERSION\n');

fprintf('Difference IF      = %.3f GHz\n',fIF/1e9);

fprintf('Difference IF      = %.0f MHz\n',fIF/1e6);

fprintf('Sum Frequency      = %.3f GHz\n',fSUM/1e9);

%% =========================================================

% CONVERT POWER FROM dBm TO WATTS

% =========================================================

PRF_W = 10^((PRF_dBm-30)/10);

PLO_W = 10^((PLO_dBm-30)/10);

fprintf('\nINPUT POWERS\n');

fprintf('RF Power = %.6f mW\n',PRF_W*1000);

fprintf('LO Power = %.6f mW\n',PLO_W*1000);

%% =========================================================

% CONVERT POWER TO RMS VOLTAGE

% ==========================================================

VRF = sqrt(PRF_W*ZIF);

VLO = sqrt(PLO_W*ZIF);

fprintf('\nRMS INPUT VOLTAGES\n');

fprintf('RF Voltage = %.4f V\n',VRF);

fprintf('LO Voltage = %.4f V\n',VLO);

%% =========================================================

% MIXER SIGNAL GENERATION

% ==========================================================

% Simulation time

fs = 200e9;             % Sampling frequency

T = 20e-9;              % Simulation duration

t = 0:1/fs:T;

% RF signal

rf_signal = VRF*sqrt(2)*cos(2*pi*fRF*t);

% LO signal

lo_signal = VLO*sqrt(2)*cos(2*pi*fLO*t);

%% =========================================================

% MIXING OPERATION

% ==========================================================

% Ideal multiplier mixer

mixed_signal = rf_signal .* lo_signal;

%% =========================================================

% THEORETICAL FREQUENCY COMPONENTS

% ==========================================================

% Product-to-sum relationship:

%

% cos(A)cos(B)

% = 1/2[cos(A-B)+cos(A+B)]

IF_signal = (VRF*VLO)*cos(2*pi*fIF*t);

SUM_signal = (VRF*VLO)*cos(2*pi*fSUM*t);

%% =========================================================

% TIME DOMAIN RF AND LO SIGNALS

% ==========================================================

figure;

plot(t*1e9,rf_signal,'LineWidth',1.5);

xlim([0 2]);

grid on;

xlabel('Time (ns)');

ylabel('RF Voltage (V)');

title('RF Input Signal - 10 GHz');

%% =========================================================

% LO SIGNAL

% ==========================================================

figure;

plot(t*1e9,lo_signal,'LineWidth',1.5);

xlim([0 2]);

grid on;

xlabel('Time (ns)');

ylabel('LO Voltage (V)');

title('LO Signal - 9.5 GHz');

%% =========================================================

% MIXER OUTPUT

% ==========================================================

figure;

plot(t*1e9,mixed_signal,'LineWidth',1.5);

xlim([0 10]);

grid on;

xlabel('Time (ns)');

ylabel('Mixer Output Voltage (V)');

title('Mixer Output Signal');

%% =========================================================

% FFT ANALYSIS

% ==========================================================

N = length(mixed_signal);

Y = fft(mixed_signal);

P2 = abs(Y/N);

P1 = P2(1:floor(N/2)+1);

P1(2:end-1) = 2*P1(2:end-1);

freq_axis = fs*(0:floor(N/2))/N;

%% =========================================================

% FREQUENCY SPECTRUM

% ==========================================================

figure;

plot(freq_axis/1e9,P1,'LineWidth',1.5);

xlim([0 20]);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude');

title('Mixer Output Frequency Spectrum');

%% =========================================================

% ZOOMED FREQUENCY SPECTRUM

% ==========================================================

figure;

plot(freq_axis/1e9,P1,'LineWidth',2);

xlim([0 11]);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude');

title('RF, LO, IF and Sum Frequency Components');

%% ========================================================

% FREQUENCY MARKERS

% ==========================================================

hold on;

xline(fIF/1e9,'--','IF = 0.5 GHz','LineWidth',1.5);

xline(fSUM/1e9,'--','SUM = 19.5 GHz','LineWidth',1.5);

%% =========================================================

% IF OUTPUT SIGNAL

% ==========================================================

figure;

plot(t*1e9,IF_signal,'LineWidth',1.5);

xlim([0 20]);

grid on;

xlabel('Time (ns)');

ylabel('IF Voltage (V)');

title('Intermediate Frequency Signal - 500 MHz');

%% =========================================================

% MIXER FREQUENCY SUMMARY

% ==========================================================

fprintf('\n');

fprintf('====================================================\n');

fprintf(' MIXER FREQUENCY COMPONENTS\n');

fprintf('====================================================\n');

fprintf('RF Frequency       = %.2f GHz\n',fRF/1e9);

fprintf('LO Frequency       = %.2f GHz\n',fLO/1e9);

fprintf('IF Difference      = %.2f GHz\n',fIF/1e9);

fprintf('IF Frequency       = %.0f MHz\n',fIF/1e6);

fprintf('Sum Frequency      = %.2f GHz\n',fSUM/1e9);

fprintf('\nThe mixer generates:\n');

fprintf('1. Difference frequency = %.2f GHz\n',fIF/1e9);

fprintf('2. Sum frequency        = %.2f GHz\n',fSUM/1e9);

fprintf('\n====================================================\n');

fprintf('CONCLUSION\n');

fprintf('====================================================\n');

fprintf('The 10 GHz RF signal and 9.5 GHz LO signal are mixed.\n');

fprintf('The desired IF output is 500 MHz.\n');

fprintf('The mixer also produces a sum-frequency component at 19.5 GHz.\n');

fprintf('\nSimulation completed successfully.\n');

Output:

Result:

The microwave semiconductor device and mixer were simulated successfully. The frequency-conversion process was analyzed, and the intermediate frequency was identified from the RF and local oscillator signals.