Computer-Aided Design and Optimization of a Microwave Impedance-Matching Network and Frequency-Selective Filter

Aim:

To design and simulate a microwave impedance-matching network and frequency-selective filter using simulation software and analyze their performance.

Procedure:

- Open MATLAB or suitable microwave simulation software.

- Design an impedance-matching network by specifying the source impedance, load impedance, and operating frequency.

- Design a frequency-selective filter with the required cutoff frequency and bandwidth.

- Simulate both circuits and plot their frequency responses and S-parameters.

- Optimize the circuit parameters to minimize reflection and improve impedance matching and filter performance.

Code:

clc;

clear;

close all;

%% 1. INPUT PARAMETERS

f0 = 10e9;                  % Design frequency = 10 GHz

Z0 = 50;                    % Reference impedance = 50 ohms

ZL = 75 + 1j*25;            % Load impedance = 75 + j25 ohms

fc = 10e9;                  % Filter cutoff frequency = 10 GHz

fstart = 8e9;               % Start frequency = 8 GHz

fstop = 12e9;               % Stop frequency = 12 GHz

f = linspace(fstart, fstop, 1001);

w0 = 2*pi*f0;

w = 2*pi*f;

%% 2. INITIAL LOAD ANALYSIS

GammaL = (ZL - Z0)/(ZL + Z0);VSWRL = (1 + abs(GammaL))/(1 - abs(GammaL));

fprintf('MICROWAVE IMPEDANCE MATCHING NETWORK\n');

fprintf('-----------------------------------\n');

fprintf('Design frequency = %.2f GHz\n', f0/1e9);

fprintf('Reference impedance = %.2f ohms\n', Z0);

fprintf('Load impedance = %.2f + j%.2f ohms\n', ...

real(ZL), imag(ZL));

fprintf('Initial reflection coefficient = %.4f %+.4fj\n', ...

real(GammaL), imag(GammaL));

fprintf('Initial |Gamma| = %.4f\n', abs(GammaL));

fprintf('Initial VSWR = %.4f\n\n', VSWRL);

%% 3. IMPEDANCE-MATCHING NETWORK OPTIMIZATION

% Topology: series reactance followed by shunt susceptance

% Optimize Xs (ohms) and Bs (siemens) at the design frequency.

objective = @(x) matching_error(x, ZL, Z0);

x0 = [40, 0.005];

options = optimset('Display','off', ...

'MaxIter',5000, ...

'MaxFunEvals',10000, ...

'TolX',1e-12, ...

'TolFun',1e-14);

xopt = fminsearch(objective, x0, options);

Xs = xopt(1);               % Series reactance at f0

Bs = xopt(2);               % Shunt susceptance at f0

% Convert reactance into a physical series inductor or capacitor

if Xs > 0

Ls = Xs/w0;

Cs = NaN;

seriesType = 'Inductor';

elseif Xs < 0

Cs = -1/(w0*Xs);

Ls = NaN;

seriesType = 'Capacitor';

else

Ls = NaN;

Cs = NaN;

seriesType = 'None';

end

% Convert susceptance into a physical shunt capacitor or inductor

if Bs > 0

Cp = Bs/w0;

Lp = NaN;

shuntType = 'Capacitor';

elseif Bs < 0

Lp = -1/(w0*Bs);

Cp = NaN;

shuntType = 'Inductor';

else

Cp = NaN;

Lp = NaN;

shuntType = 'None';

end

%% 4. FREQUENCY SWEEP OF MATCHING NETWORK

% Frequency-dependent series reactance

if Xs > 0

Xseries = w*Ls;

elseif Xs < 0

Xseries = -1./(w*Cs);

else

Xseries = zeros(size(f));

end

% Frequency-dependent shunt susceptance

if Bs > 0

Bshunt = w*Cp;

elseif Bs < 0

Bshunt = -1./(w*Lp);

else

Bshunt = zeros(size(f));

end

Zseries = ZL + 1j*Xseries;

Yin = 1./Zseries + 1j*Bshunt;

Zin = 1./Yin;

GammaIn = (Zin - Z0)./(Zin + Z0);

S11_dB = 20*log10(max(abs(GammaIn),1e-12));

VSWR = (1 + abs(GammaIn))./(1 - abs(GammaIn));

% Exact design-frequency result

Zseries0 = ZL + 1j*Xs;

Zin0 = 1/(1/Zseries0 + 1j*Bs);

Gamma0 = (Zin0 - Z0)/(Zin0 + Z0);

VSWR0 = (1 + abs(Gamma0))/(1 - abs(Gamma0));

fprintf('OPTIMIZED MATCHING NETWORK\n');

fprintf('--------------------------\n');

fprintf('Series element type: %s\n', seriesType);

fprintf('Series reactance = %.4f ohms\n', Xs);

if isfinite(Ls)

fprintf('Series inductance = %.4f pH\n', Ls*1e12);

end

if isfinite(Cs)

fprintf('Series capacitance = %.4f fF\n', Cs*1e15);

end

fprintf('Shunt element type: %s\n', shuntType);

fprintf('Shunt susceptance = %.6f S\n', Bs);

if isfinite(Cp)

fprintf('Shunt capacitance = %.4f fF\n', Cp*1e15);

end

if isfinite(Lp)

fprintf('Shunt inductance = %.4f pH\n', Lp*1e12);

end

fprintf('\nMatched input impedance at 10 GHz:\n');

fprintf('Zin = %.4f %+.4fj ohms\n', real(Zin0), imag(Zin0));

fprintf('|Gamma| = %.6f\n', abs(Gamma0));

fprintf('Return loss = %.4f dB\n', -20*log10(max(abs(Gamma0),1e-12)));

fprintf('VSWR = %.4f\n\n', VSWR0);

%% 5. THIRD-ORDER BUTTERWORTH LOW-PASS FILTER

% Ideal normalized Butterworth magnitude response

N = 3;

Hmag = 1./sqrt(1 + (f/fc).^(2*N));

H_dB = 20*log10(max(Hmag,1e-12));

fprintf('FREQUENCY-SELECTIVE FILTER\n');

fprintf('--------------------------\n');

fprintf('Filter type = Third-order Butterworth low-pass\n');

fprintf('Cutoff frequency = %.2f GHz\n', fc/1e9);

fprintf('Sweep range = %.2f to %.2f GHz\n', ...

fstart/1e9, fstop/1e9);

fprintf('Ideal attenuation at cutoff = %.2f dB\n', ...

20*log10(1/sqrt(2)));

%% 6. PLOT INPUT REFLECTION COEFFICIENT

figure;

plot(f/1e9, abs(GammaIn), 'b', 'LineWidth', 2);

grid on;

xlabel('Frequency (GHz)');

ylabel('|S_{11}|');

title('Reflection Coefficient of Matching Network');

xline(f0/1e9, '--r', 'Design Frequency');

%% 7. PLOT RETURN LOSS

figure;

plot(f/1e9, S11_dB, 'r', 'LineWidth', 2);

grid on;

xlabel('Frequency (GHz)');

ylabel('S_{11} (dB)');

title('Return Loss of Matching Network');

xline(f0/1e9, '--k', '10 GHz');

%% 8. PLOT VSWR

figure;

plot(f/1e9, VSWR, 'm', 'LineWidth', 2);

grid on;

xlabel('Frequency (GHz)');

ylabel('VSWR');

title('VSWR versus Frequency');

xline(f0/1e9, '--k', 'Design Frequency');

%% 9. PLOT FILTER FREQUENCY RESPONSE

figure;

plot(f/1e9, H_dB, 'LineWidth', 2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

title('Third-Order Butterworth Low-Pass Filter Response');

xline(fc/1e9, '--r', 'Cutoff = 10 GHz');

yline(-3.0103, '--k', '-3 dB');

%% 10. PLOT INPUT IMPEDANCE

figure;

plot(f/1e9, real(Zin), 'b', 'LineWidth', 2);

hold on;

plot(f/1e9, imag(Zin), 'r--', 'LineWidth', 2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Impedance (ohms)');

title('Input Impedance of Matching Network');

legend('Real(Zin)', 'Imag(Zin)', 'Location', 'best');

xline(f0/1e9, ':k', '10 GHz');

%% LOCAL OBJECTIVE FUNCTION

function err = matching_error(x, ZL, Z0)

Xs = x(1);

Bs = x(2);

Zseries = ZL + 1j*Xs;

Yin = 1/Zseries + 1j*Bs;

Zin = 1/Yin;

Gamma = (Zin - Z0)/(Zin + Z0);

err = abs(Gamma)^2;

if ~isfinite(err)

err = 1e10;

end

end

Output:

Result:

The microwave impedance-matching network and frequency-selective filter were designed and simulated successfully. Their performance was analyzed using S-parameters and frequency-response plots, and suitable parameters were optimized to improve impedance matching and frequency selectivity.