Simulation-Based Electromagnetic Analysis of TEM, TE and TM Propagation Modes in Microwave Transmission Structures

Aim:

To simulate and analyze the electromagnetic field distribution and propagation characteristics of TEM, TE, and TM modes in microwave transmission structures.

Procedure:

- Open the electromagnetic simulation software, such as MATLAB or CST Studio Suite.

- Select a suitable microwave transmission structure, such as a waveguide or coaxial cable.

- Define the structure dimensions, operating frequency, and material properties.

- Simulate the TEM, TE, and TM propagation modes and observe their electric and magnetic field distributions.

- Analyze the propagation characteristics and compare the results of the three modes.

Code:

clc;

clear;

close all;

%% =========================================================

%  ELECTROMAGNETIC MODE ANALYSIS OF RECTANGULAR WAVEGUIDE

%  TE10 AND TM11 MODES

% ==========================================================

%% Input Parameters

c = 3e8;                    % Speed of light (m/s)

f = 10e9;                   % Operating frequency (Hz)

a = 22.86e-3;               % Width (m)

b = 10.16e-3;               % Height (m)

L = 100e-3;                 % Waveguide length (m)

fprintf('\n============================================\n');

fprintf(' RECTANGULAR WAVEGUIDE MODE ANALYSIS\n');

fprintf('============================================\n');

fprintf('Operating Frequency : %.2f GHz\n',f/1e9);

fprintf('Waveguide Width     : %.2f mm\n',a*1000);

fprintf('Waveguide Height    : %.2f mm\n',b*1000);

fprintf('Waveguide Length    : %.2f mm\n',L*1000);

%% =========================================================

% TE10 MODE

% ==========================================================

m1 = 1;

n1 = 0;

fc_TE10 = (c/2)*sqrt((m1/a)^2+(n1/b)^2);

k0 = 2*pi*f/c;

if f >= fc_TE10

beta_TE10 = sqrt(k0^2-(2*pi*fc_TE10/c)^2);

lambda_g = 2*pi/beta_TE10;

TE10_status = 'PROPAGATING';

else

beta_TE10 = 0;

lambda_g = Inf;

TE10_status = 'CUT OFF';

end

%% =========================================================

% TM11 MODE

% ==========================================================

m2 = 1;

n2 = 1;

fc_TM11 = (c/2)*sqrt((m2/a)^2+(n2/b)^2);

if f >= fc_TM11

beta_TM11 = sqrt(k0^2-(2*pi*fc_TM11/c)^2);

TM11_status = 'PROPAGATING';

else

alpha_TM11 = sqrt((2*pi*fc_TM11/c)^2-k0^2);

beta_TM11 = 0;

TM11_status = 'CUT OFF / EVANESCENT';

end

%% =========================================================

% DISPLAY RESULTS

% ==========================================================

fprintf('\n--------------------------------------------\n');

fprintf('TE10 MODE RESULTS\n');

fprintf('--------------------------------------------\n');

fprintf('Cutoff Frequency : %.3f GHz\n',fc_TE10/1e9);

fprintf('Propagation Constant : %.3f rad/m\n',beta_TE10);

fprintf('Guide Wavelength : %.3f mm\n',lambda_g*1000);

fprintf('Mode Status : %s\n',TE10_status);

fprintf('\n--------------------------------------------\n');

fprintf('TM11 MODE RESULTS\n');

fprintf('--------------------------------------------\n');

fprintf('Cutoff Frequency : %.3f GHz\n',fc_TM11/1e9);

fprintf('Mode Status : %s\n',TM11_status);

if f < fc_TM11

fprintf('Attenuation Constant : %.3f Np/m\n',alpha_TM11);

end

%% =========================================================

% FIELD GRID

% ==========================================================

Nx = 150;

Ny = 100;

x = linspace(0,a,Nx);

y = linspace(0,b,Ny);

[X,Y] = meshgrid(x,y);

%% =========================================================

% TE10 ELECTRIC FIELD

% ==========================================================

E_TE10 = sin(pi*X/a);

figure;

imagesc(x*1000,y*1000,E_TE10);

axis xy;

axis equal tight;

colorbar;

xlabel('x (mm)');

ylabel('y (mm)');

title('TE_{10} Electric Field Distribution');

%% =========================================================

% TE10 MAGNETIC FIELD

% ==========================================================

H_TE10 = cos(pi*X/a);

figure;

imagesc(x*1000,y*1000,H_TE10);

axis xy;

axis equal tight;

colorbar;

xlabel('x (mm)');

ylabel('y (mm)');

title('TE_{10} Magnetic Field Distribution');

%% =========================================================

% TM11 ELECTRIC FIELD

% ==========================================================

E_TM11 = sin(pi*X/a).*sin(pi*Y/b);

figure;

imagesc(x*1000,y*1000,E_TM11);

axis xy;

axis equal tight;

colorbar;

xlabel('x (mm)');

ylabel('y (mm)');

title('TM_{11} Electric Field Distribution');

%% =========================================================

% TM11 MAGNETIC FIELD

% ==========================================================

H_TM11 = cos(pi*X/a).*cos(pi*Y/b);

figure;

imagesc(x*1000,y*1000,H_TM11);

axis xy;

axis equal tight;

colorbar;

xlabel('x (mm)');

ylabel('y (mm)');

title('TM_{11} Magnetic Field Distribution');

%% =========================================================

% FIELD PROPAGATION ALONG WAVEGUIDE

% =========================================================

z = linspace(0,L,300);

% TE10 propagating field

TE10_wave = cos(beta_TE10*z);

figure;

plot(z*1000,TE10_wave,'LineWidth',2);

grid on;

xlabel('Waveguide Length (mm)');

ylabel('Normalized Field');

title('TE_{10} Propagation Along Waveguide');

%% =========================================================

% TM11 EVANESCENT FIELD

% ==========================================================

if f < fc_TM11

TM11_wave = exp(-alpha_TM11*z);

figure;

plot(z*1000,TM11_wave,'LineWidth',2);

grid on;

xlabel('Waveguide Length (mm)');

ylabel('Normalized Field');

title('TM_{11} Evanescent Field');

end

%% =========================================================

% CUTOFF FREQUENCY COMPARISON

% ==========================================================

figure;

bar([fc_TE10 fc_TM11]/1e9);

set(gca,'XTickLabel',{'TE10','TM11'});

ylabel('Cutoff Frequency (GHz)');

xlabel('Mode');

title('Cutoff Frequency Comparison');

grid on;

%% =========================================================

% OPERATING FREQUENCY COMPARISON

% ==========================================================

figure;

bar([f fc_TE10 fc_TM11]/1e9);

set(gca,'XTickLabel', ...

{'Operating Frequency','TE10 Cutoff','TM11 Cutoff'});

ylabel('Frequency (GHz)');

title('Operating Frequency and Cutoff Frequencies');

grid on;

%% =========================================================

% FINAL RESULT

% ==========================================================

fprintf('\n============================================\n');

fprintf('FINAL OBSERVATION\n');

fprintf('============================================\n');

fprintf('At %.2f GHz:\n',f/1e9);

if f > fc_TE10

fprintf('TE10 : PROPAGATES\n');

else

fprintf('TE10 : DOES NOT PROPAGATE\n');

end

if f > fc_TM11

fprintf('TM11 : PROPAGATES\n');

else

fprintf('TM11 : DOES NOT PROPAGATE\n');

end

fprintf('\nTE10 is the dominant usable mode at 10 GHz.\n');

fprintf('TM11 is below cutoff and is therefore evanescent.\n');

fprintf('\nSimulation completed successfully.\n');

Output:

+

Result:

The TEM, TE, and TM propagation modes were simulated and analyzed successfully. Their electromagnetic field distributions and propagation characteristics were studied, and the differences between the three modes were identified.