Numerical Simulation and Performance Analysis of Microwave Transmission Lines and Two-Port Networks Using Scattering Parameters

Aim:

To simulate microwave transmission lines and two-port networks and analyze their performance using scattering parameters (S-parameters).

Procedure:

- Open MATLAB or suitable microwave simulation software.

- Define the transmission line parameters, operating frequency, characteristic impedance, and load impedance.

- Calculate the S-parameters (S11, S21, S12, and S22) for the two-port network.

- Simulate and plot the S-parameters over the selected frequency range.

- Analyze the reflection, transmission, return loss, and insertion loss to evaluate the network performance.

Code:

clc;

clear;

close all;

%% =========================================================

% NUMERICAL SIMULATION OF MICROWAVE TRANSMISSION LINE

% USING SCATTERING PARAMETERS

% ==========================================================

%% INPUT PARAMETERS

c = 3e8;                    % Speed of light (m/s)

f0 = 10e9;                  % Operating frequency = 10 GHz

Z0 = 50;                    % Reference impedance = 50 ohm

fstart = 8e9;               % Start frequency = 8 GHz

fstop  = 12e9;              % Stop frequency = 12 GHz

L = 50e-3;                  % Network length = 50 mm

ZL = 50;                    % Load impedance = 50 ohm

%% Frequency sweep

N = 501;

f = linspace(fstart,fstop,N);

omega = 2*pi*f;

%% =========================================================

% TRANSMISSION LINE PARAMETERS

% ==========================================================

% Lossless transmission line

beta = omega/c;

% Electrical length

theta = beta*L;

%% =========================================================

% INPUT IMPEDANCE

% ==========================================================

% General transmission-line input impedance

Zin = Z0 .* (ZL + 1i*Z0.*tan(theta)) ./ ...

(Z0 + 1i*ZL.*tan(theta));

%% =========================================================

% SCATTERING PARAMETERS

% ==========================================================

% Reflection coefficient at input

S11 = (Zin-Z0)./(Zin+Z0);

% For a reciprocal lossless transmission line

S22 = S11;

% Transmission coefficient

S21 = 2*ZL ./ ...

(ZL + Z0 + (ZL-Z0).*exp(-2i*theta));

% For reciprocal network

S12 = S21;

%% =========================================================

% MAGNITUDE OF S PARAMETERS

% ==========================================================

S11_mag = abs(S11);

S21_mag = abs(S21);

S12_mag = abs(S12);

S22_mag = abs(S22);

%% Convert to dB

S11_dB = 20*log10(max(S11_mag,1e-12));

S21_dB = 20*log10(max(S21_mag,1e-12));

S12_dB = 20*log10(max(S12_mag,1e-12));

S22_dB = 20*log10(max(S22_mag,1e-12));

%% Phase

S11_phase = unwrap(angle(S11))*180/pi;

S21_phase = unwrap(angle(S21))*180/pi;

%% Return Loss

ReturnLoss = -20*log10(max(S11_mag,1e-12));

%% Insertion Loss

InsertionLoss = -20*log10(max(S21_mag,1e-12));

%% =========================================================

% DISPLAY RESULTS AT 10 GHz

% ==========================================================

[~,index] = min(abs(f-f0));

fprintf('\n');

fprintf('====================================================\n');

fprintf(' MICROWAVE TRANSMISSION LINE S-PARAMETER ANALYSIS\n');

fprintf('====================================================\n');

fprintf('\nINPUT PARAMETERS\n');

fprintf('Operating Frequency : %.2f GHz\n',f0/1e9);

fprintf('Reference Impedance : %.2f Ohm\n',Z0);

fprintf('Frequency Range     : %.2f - %.2f GHz\n',...

fstart/1e9,fstop/1e9);

fprintf('Network Length      : %.2f mm\n',L*1000);

fprintf('Load Impedance      : %.2f Ohm\n',ZL);

fprintf('\n----------------------------------------------------\n');

fprintf('RESULTS AT %.2f GHz\n',f0/1e9);

fprintf('----------------------------------------------------\n');

fprintf('S11 Magnitude       : %.4f\n',S11_mag(index));

fprintf('S11 (dB)            : %.4f dB\n',S11_dB(index));

fprintf('S21 Magnitude       : %.4f\n',S21_mag(index));

fprintf('S21 (dB)            : %.4f dB\n',S21_dB(index));

fprintf('S12 Magnitude       : %.4f\n',S12_mag(index));

fprintf('S12 (dB)            : %.4f dB\n',S12_dB(index));

fprintf('S22 Magnitude       : %.4f\n',S22_mag(index));

fprintf('S22 (dB)            : %.4f dB\n',S22_dB(index));

fprintf('Return Loss         : %.4f dB\n',ReturnLoss(index));

fprintf('Insertion Loss      : %.4f dB\n',InsertionLoss(index));

fprintf('Phase of S21        : %.2f degrees\n',S21_phase(index));

%% =========================================================

% PLOT 1: S11 AND S21 MAGNITUDE

% ==========================================================

figure;

plot(f/1e9,S11_mag,'LineWidth',2);

hold on;

plot(f/1e9,S21_mag,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude');

title('S-Parameter Magnitude');

legend('|S_{11}|','|S_{21}|','Location','best');

%% =========================================================

% PLOT 2: S PARAMETERS IN dB

% ==========================================================

figure;

plot(f/1e9,S11_dB,'LineWidth',2);

hold on;

plot(f/1e9,S21_dB,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

title('S-Parameters in dB');

legend('S_{11}','S_{21}','Location','best');

%% =========================================================

% PLOT 3: S11 PHASE

% ==========================================================

figure;

plot(f/1e9,S11_phase,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Phase (degrees)');

title('S_{11} Phase Response');

%% =========================================================

% PLOT 4: S21 PHASE

% ==========================================================

figure;

plot(f/1e9,S21_phase,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)';

ylabel('Phase (degrees)');

title('S_{21} Phase Response');

%% =========================================================

% PLOT 5: RETURN LOSS

% ==========================================================

figure;

plot(f/1e9,ReturnLoss,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Return Loss (dB)')

title('Return Loss vs Frequency');

%% =========================================================

% PLOT 6: INSERTION LOSS

% ==========================================================

figure;

plot(f/1e9,InsertionLoss,'LineWidth',2);

grid on;

Output:

Result:

The microwave transmission lines and two-port networks were simulated successfully. The S-parameters were calculated and analyzed to determine the reflection and transmission characteristics and evaluate the overall performance of the microwave network.