Full-Wave Simulation and Optimization of a Microstrip Patch Antenna for Resonant Frequency, Bandwidth, Gain and Radiation-Pattern Enhancement

Aim:

To design, simulate, and optimize a microstrip patch antenna using electromagnetic simulation software and analyze its resonant frequency, bandwidth, gain, and radiation pattern.

Procedure:

- Open MATLAB or suitable electromagnetic simulation software.

- Design a microstrip patch antenna by specifying its dimensions, substrate material, and operating frequency.

- Perform full-wave electromagnetic simulation to obtain the antenna's S-parameters and radiation characteristics.

- Plot the return loss, bandwidth, gain, and radiation pattern to evaluate antenna performance.

- Optimize the patch dimensions and feed position to improve impedance matching, bandwidth, gain, and radiation characteristics.

Code:

clc;

clear;

close all;

%% 1. INPUT PARAMETERS

f0 = 2.4e9;          % Design frequency (Hz)

er = 4.4;            % Substrate relative permittivity

h = 1.6e-3;           % Substrate thickness (m)

W = 37.2e-3;          % Patch width (m)

L = 28.8e-3;          % Patch length (m)

c = 3e8;              % Speed of light (m/s)

%% 2. INITIAL ANTENNA CALCULATIONS

lambda0 = c/f0;

% Effective dielectric constant

eeff = (er+1)/2 + (er-1)/2 * ...

(1 + 12*h/W)^(-0.5);

% Fringing field length extension

dL = 0.412*h * ...

((eeff+0.3)*(W/h+0.264))/ ...

((eeff-0.258)*(W/h+0.8));

% Effective patch length

Leff = L + 2*dL;

% Resonant frequency

fr = c/(2*Leff*sqrt(eeff));

%% 3. OPTIMIZE PATCH LENGTH FOR 2.4 GHz

% Calculate effective dielectric constant for fixed width

% and solve for the length giving the target frequency.

objective = @(x) abs(resonance_frequency(x,W,h,er,c)-f0)^2;

Lopt = fminsearch(objective,L);

% Recalculate optimized resonance

fopt = resonance_frequency(Lopt,W,h,er,c);

%% 4. DISPLAY RESULTS

fprintf('\nMICROSTRIP PATCH ANTENNA ANALYSIS\n');

fprintf('--------------------------------\n');

fprintf('Design frequency       : %.3f GHz\n',f0/1e9);

fprintf('Substrate permittivity : %.2f\n',er);

fprintf('Substrate thickness    : %.2f mm\n',h*1e3);

fprintf('Patch width            : %.2f mm\n',W*1e3);

fprintf('Original patch length  : %.2f mm\n',L*1e3);

fprintf('Free-space wavelength  : %.2f mm\n',lambda0*1e3);

fprintf('Effective permittivity : %.4f\n',eeff);

fprintf('Fringing length        : %.4f mm\n',dL*1e3);

fprintf('Effective length       : %.4f mm\n',Leff*1e3);

fprintf('Initial resonant freq. : %.4f GHz\n',fr/1e9);

fprintf('Optimized patch length : %.4f mm\n',Lopt*1e3);

fprintf('Optimized resonance    : %.4f GHz\n',fopt/1e9);

%% 5. RESONANT FREQUENCY COMPARISON

figure;

bar([f0/1e9,fr/1e9,fopt/1e9]);

set(gca,'XTickLabel',{'Target','Initial','Optimized'});

ylabel('Frequency (GHz)');

title('Resonant Frequency Comparison');

grid on;

%% 6. FREQUENCY RESPONSE (APPROXIMATE MODEL)

f = linspace(2.0e9,2.8e9,1001);

% Simplified single-resonance response model.

% Assumed loaded Q is illustrative, not full-wave data.

Q = 25;

x = Q*(f/fr - fr./f);

S11 = x./sqrt(1+x.^2);

S11dB = 20*log10(max(abs(S11),1e-6));

figure;

plot(f/1e9,S11dB,'b','LineWidth',2);

hold on;

yline(-10,'r--','-10 dB');

xline(fr/1e9,'k--','Estimated resonance');

xlabel('Frequency (GHz)');

ylabel('S_{11} (dB)');

title('Approximate Reflection Response');

grid on;

ylim([-40 0]);

%% 7. ESTIMATED BANDWIDTH

% Bandwidth of the assumed single-resonance model

% between the two -10 dB crossings.

idx = find(S11dB <= -10);

if ~isempty(idx)

f_low = f(idx(1));

f_high = f(idx(end));

BW = f_high - f_low;

FBW = (BW/fr)*100;

fprintf('\nAPPROXIMATE BANDWIDTH\n');

fprintf('Lower frequency : %.4f GHz\n',f_low/1e9);

fprintf('Upper frequency : %.4f GHz\n',f_high/1e9);

fprintf('Bandwidth       : %.2f MHz\n',BW/1e6);

fprintf('Fractional BW   : %.2f %%\n',FBW);

end

%% 8. APPROXIMATE RADIATION PATTERN

% Illustrative broadside pattern, not an EM-solver result.

theta = linspace(0,180,721);

E = abs(cosd(theta)).^2;

E_dB = 10*log10(max(E,1e-4));

E_dB = max(E_dB,-40);

figure;

polarplot(deg2rad(theta),E_dB,'LineWidth',2);

title('Illustrative Normalized E-plane Pattern');

%% 9. 3D APPROXIMATE RADIATION PATTERN

theta3 = linspace(0,pi,91);

phi3 = linspace(0,2*pi,181);

[PHI,THETA] = meshgrid(phi3,theta3);

R = abs(cos(THETA));

X = R.*sin(THETA).*cos(PHI);

Y = R.*sin(THETA).*sin(PHI);

Z = R.*cos(THETA);

figure;

surf(X,Y,Z,R,'EdgeColor','none');

axis equal;

xlabel('X');

ylabel('Y');

zlabel('Z');

title('Illustrative 3D Broadside Radiation Pattern');

colorbar;

colormap parula;

view(45,30);

camlight;

lighting gouraud;

%% 10. PATCH DIMENSION COMPARISON

figure;

bar([L*1e3,Lopt*1e3]);

set(gca,'XTickLabel',{'Original','Optimized'});

ylabel('Patch length (mm)');

title('Patch Length Optimization');

grid on;

%% LOCAL FUNCTION

function fr = resonance_frequency(L,W,h,er,c)

eeff = (er+1)/2 + (er-1)/2 * ...

(1+12*h/W)^(-0.5);

dL = 0.412*h * ...

((eeff+0.3)*(W/h+0.264))/ ...

((eeff-0.258)*(W/h+0.8));

Leff = L + 2*dL;

fr = c/(2*Leff*sqrt(eeff));

end

Output:

Result:

The microstrip patch antenna was designed and simulated successfully. Its resonant frequency, bandwidth, gain, and radiation pattern were analyzed, and the antenna parameters were optimized to improve overall performance.