Simulation-Based Frequency-Domain Characterization of a Microwave Two-Port Network Using S-Parameters and Network-Analyzer Emulation

Aim:

To simulate and characterize a microwave two-port network in the frequency domain using S-parameters and emulate a vector network analyzer (VNA) to evaluate its reflection and transmission characteristics.

Procedure:

- Open MATLAB or suitable microwave simulation software.

- Define the two-port network parameters, operating frequency range, characteristic impedance, and network configuration.

- Calculate or simulate the S-parameters: \(S_{11}\), \(S_{21}\), \(S_{12}\), and \(S_{22}\).

- Emulate a vector network analyzer by sweeping the frequency and displaying the magnitude and phase of the S-parameters.

- Plot the S-parameters against frequency and analyze return loss, insertion loss, transmission, and impedance matching.

- Evaluate the network performance and compare the simulated results with the expected theoretical characteristics.

Code:

clc;

clear;

close all;

%% 1. INPUT PARAMETERS

fstart = 8e9;        % Start frequency = 8 GHz

fstop  = 12e9;       % Stop frequency = 12 GHz

Z0     = 50;         % Reference impedance = 50 ohms

df     = 0.01e9;     % Frequency step = 0.01 GHz

% Frequency vector

f = (fstart:df:fstop).';

N = length(f);

%% 2. TWO-PORT NETWORK MODEL

% Example DUT: matched 50-ohm transmission line

% with frequency-dependent attenuation and phase.

% These parameters are illustrative and can be changed.

Zc = 50;                  % Characteristic impedance (ohms)

lineLength = 0.03;        % Transmission line length (m)

c = 3e8;                  % Speed of light (m/s)

vf = 0.7;                 % Assumed velocity factor

alpha_dB_per_m = 10;      % Assumed attenuation (dB/m)

% Allocate S-parameter arrays

S11 = zeros(N,1);

S21 = zeros(N,1);

S12 = zeros(N,1);

S22 = zeros(N,1);

for k = 1:N

freq = f(k);

% Propagation constant

alpha = alpha_dB_per_m/8.686;  % Np/m

beta = 2*pi*freq/(vf*c);

gamma = alpha + 1j*beta;

% Transmission-line ABCD parameters

A = cosh(gamma*lineLength);

B = Zc*sinh(gamma*lineLength);

C = sinh(gamma*lineLength)/Zc;

D = cosh(gamma*lineLength);

% Convert ABCD parameters to S-parameters

denominator = A + B/Z0 + C*Z0 + D;

S11(k) = (A + B/Z0 - C*Z0 - D)/denominator;

S21(k) = 2/denominator;

S12(k) = 2*(A*D-B*C)/denominator;

S22(k) = (-A + B/Z0 - C*Z0 + D)/denominator;

end

%% 3. MAGNITUDE IN dB AND PHASE

S11_dB = 20*log10(max(abs(S11),1e-12));

S21_dB = 20*log10(max(abs(S21),1e-12));

S12_dB = 20*log10(max(abs(S12),1e-12));

S22_dB = 20*log10(max(abs(S22),1e-12));

S11_phase = unwrap(angle(S11))*180/pi;

S21_phase = unwrap(angle(S21))*180/pi;

S12_phase = unwrap(angle(S12))*180/pi;

S22_phase = unwrap(angle(S22))*180/pi;

%% 4. DISPLAY SUMMARY

fprintf('MICROWAVE TWO-PORT NETWORK ANALYSIS\n');

fprintf('-----------------------------------\n');

fprintf('Start frequency     : %.2f GHz\n',fstart/1e9);

fprintf('Stop frequency      : %.2f GHz\n',fstop/1e9);

fprintf('Reference impedance : %.2f ohms\n',Z0);

fprintf('Frequency step      : %.3f GHz\n',df/1e9);

fprintf('Number of points    : %d\n',N);

fprintf('DUT type             : Transmission line\n');

fprintf('Line length          : %.2f mm\n',lineLength*1e3);

% Find the data point closest to 10 GHz

[~,idx10] = min(abs(f-10e9));

fprintf('\nS-PARAMETERS AT %.2f GHz\n',f(idx10)/1e9);

fprintf('S11 = %.4f %+.4fj\n',real(S11(idx10)),imag(S11(idx10)));

fprintf('S21 = %.4f %+.4fj\n',real(S21(idx10)),imag(S21(idx10)));

fprintf('S12 = %.4f %+.4fj\n',real(S12(idx10)),imag(S12(idx10)));

fprintf('S22 = %.4f %+.4fj\n',real(S22(idx10)),imag(S22(idx10)));

fprintf('\nS11 magnitude = %.2f dB\n',S11_dB(idx10));

fprintf('S21 magnitude = %.2f dB\n',S21_dB(idx10));

fprintf('S12 magnitude = %.2f dB\n',S12_dB(idx10));

fprintf('S22 magnitude = %.2f dB\n',S22_dB(idx10));

fprintf('\nReturn loss at 10 GHz = %.2f dB\n',-S11_dB(idx10));

fprintf('Insertion loss at 10 GHz = %.2f dB\n',-S21_dB(idx10));

%% 5. PLOT S-PARAMETER MAGNITUDES

figure;

plot(f/1e9,S11_dB,'LineWidth',1.8);

hold on;

plot(f/1e9,S21_dB,'LineWidth',1.8);

plot(f/1e9,S12_dB,'LineWidth',1.8);

plot(f/1e9,S22_dB,'LineWidth',1.8);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

title('Two-Port Network S-Parameter Magnitudes');

legend('S11','S21','S12','S22','Location','best');

%% 6. PLOT RETURN LOSS

figure;

plot(f/1e9,-S11_dB,'b','LineWidth',1.8);

hold on;

plot(f/1e9,-S22_dB,'r--','LineWidth',1.8);

grid on;

xlabel('Frequency (GHz)');

ylabel('Return Loss (dB)');

title('Input and Output Return Loss');

legend('Input Return Loss','Output Return Loss','Location','best');

%% 7. PLOT INSERTION LOSS

figure;

plot(f/1e9,-S21_dB,'m','LineWidth',1.8);

grid on;

xlabel('Frequency (GHz)');

ylabel('Insertion Loss (dB)');

title('Forward Transmission Insertion Loss');

%% 8. PLOT PHASE RESPONSE

figure;

plot(f/1e9,S11_phase,'LineWidth',1.5);

hold on;

plot(f/1e9,S21_phase,'LineWidth',1.5);

plot(f/1e9,S12_phase,'LineWidth',1.5);

plot(f/1e9,S22_phase,'LineWidth',1.5);

grid on;

xlabel('Frequency (GHz)');

ylabel('Phase (degrees)');

title('Two-Port Network Phase Response');

legend('Phase S11','Phase S21','Phase S12','Phase S22', ...

'Location','best');

%% 9. SMITH CHART

figure;

smithplot(S11);

title('Input Reflection Coefficient on Smith Chart');

%% 10. TRANSMISSION CHECK

% For a passive network, transmitted plus reflected

% power should not exceed incident power.

powerSum = abs(S11).^2 + abs(S21).^2;

figure;

plot(f/1e9,powerSum,'k','LineWidth',1.8);

grid on;

xlabel('Frequency (GHz)');

ylabel('|S11|^2 + |S21|^2');

title('Input Power Balance Check');

yline(1,'r--','Unity');

fprintf('\nSimulation completed successfully.\n');

Output:

Result:

The microwave two-port network was simulated and characterized successfully using S-parameters. Frequency-dependent reflection and transmission characteristics were analyzed, and vector network analyzer emulation was performed to evaluate the network's overall microwave performance.