Numerical Simulation and Spectral Analysis of a Microwave Signal for Frequency-Domain Characterization Using Spectrum-Analyzer Concepts

Aim:

To simulate a microwave signal and analyze its frequency spectrum using numerical methods and spectrum-analyzer concepts to determine its frequency components, bandwidth, and spectral characteristics.

Procedure:

- Open MATLAB or suitable signal-processing simulation software.

- Generate a microwave signal with a specified carrier frequency, amplitude, and sampling frequency.

- Introduce noise or additional frequency components to study their effects on the signal spectrum.

- Apply the Fast Fourier Transform (FFT) to convert the time-domain signal into the frequency domain.

- Plot the frequency spectrum and analyze the carrier frequency, harmonics, bandwidth, and noise components.

- Emulate a spectrum analyzer by displaying signal power versus frequency and evaluate the spectral characteristics.

Code:

clc;

clear;

close all;

%% 1. INPUT PARAMETERS

fc = 10e9;       % Carrier frequency = 10 GHz

A = 1;           % Carrier amplitude = 1 V

fm = 0.1e9;      % Modulating frequency = 0.1 GHz

mu = 0.5;        % Modulation index = 0.5

fs = 50e9;       % Sampling frequency = 50 GHz

%% 2. TIME VECTOR

Tsim = 200e-9;   % Simulation duration = 200 ns

N = round(fs*Tsim);

t = (0:N-1)/fs;

%% 3. GENERATE AM SIGNAL

% Standard amplitude modulation

m = cos(2*pi*fm*t);

carrier = A*cos(2*pi*fc*t);

signal_AM = A*(1 + mu*m).*cos(2*pi*fc*t);

%% 4. TIME-DOMAIN PLOT

% Display first 5 ns for clear visualization

idx = t <= 5e-9;

figure;

plot(t(idx)*1e9,signal_AM(idx),'b','LineWidth',1.2);

grid on;

xlabel('Time (ns)');

ylabel('Amplitude (V)');

title('Time-Domain AM Microwave Signal');

%% 5. FFT SPECTRAL ANALYSIS

Y = fft(signal_AM);

P2 = abs(Y/N);

P1 = P2(1:floor(N/2)+1);

P1(2:end-1) = 2*P1(2:end-1);

f = fs*(0:floor(N/2))/N;

% Convert voltage amplitude to dBV

P_dBV = 20*log10(max(P1,1e-12));

%% 6. FREQUENCY SPECTRUM

figure;

plot(f/1e9,P_dBV,'LineWidth',1.3);

grid on;

xlabel('Frequency (GHz)');

ylabel('Amplitude (dBV)');

title('Single-Sided FFT Spectrum of AM Signal');

xlim([9.5 10.5]);

%% 7. IDENTIFY CARRIER AND SIDEBANDS

f_lower = fc-fm;

f_upper = fc+fm;

fprintf('\nMICROWAVE SIGNAL SPECTRAL ANALYSIS\n');

fprintf('---------------------------------\n');

fprintf('Carrier frequency       : %.2f GHz\n',fc/1e9);

fprintf('Carrier amplitude       : %.2f V\n',A);

fprintf('Modulating frequency    : %.2f GHz\n',fm/1e9);

fprintf('Modulation index        : %.2f\n',mu);

fprintf('Sampling frequency      : %.2f GHz\n',fs/1e9);

fprintf('Number of samples       : %d\n',N);

fprintf('FFT frequency resolution: %.2f MHz\n',fs/N/1e6);

fprintf('\nEXPECTED SPECTRAL COMPONENTS\n');

fprintf('Carrier frequency       : %.2f GHz\n',fc/1e9);

fprintf('Lower sideband          : %.2f GHz\n',f_lower/1e9);

fprintf('Upper sideband          : %.2f GHz\n',f_upper/1e9);

%% 8. THEORETICAL AMPLITUDES

Ac = A;

Aside = mu*A/2;

fprintf('\nTHEORETICAL AMPLITUDES\n');

fprintf('Carrier amplitude       : %.3f V peak\n',Ac);

fprintf('Each sideband amplitude : %.3f V peak\n',Aside);

fprintf('Each sideband relative to carrier: %.2f dBc\n', ...

20*log10(Aside/Ac));

%% 9. POWER DISTRIBUTION

% Assuming a 1-ohm reference resistance

Pc = Ac^2/2;

Psb = Aside^2/2;

Ptotal = Pc + 2*Psb;

fprintf('\nPOWER ANALYSIS (1-ohm reference)\n');

fprintf('Carrier power           : %.4f W\n',Pc);

fprintf('Each sideband power     : %.4f W\n',Psb);

fprintf('Total AM signal power   : %.4f W\n',Ptotal);

%% 10. SPECTRUM NEAR CARRIER

figure;

plot(f/1e9,P_dBV,'r','LineWidth',1.3);

grid on;

xlabel('Frequency (GHz)');

ylabel('Amplitude (dBV)');

title('Detailed Microwave Spectrum');

xlim([9.8 10.2]);

hold on;

xline(f_lower/1e9,'--k','Lower Sideband');

xline(fc/1e9,'--b','Carrier');

xline(f_upper/1e9,'--k','Upper Sideband');

%% 11. AM ENVELOPE

envelope = A*(1 + mu*cos(2*pi*fm*t));

figure;

plot(t*1e9,envelope,'r','LineWidth',1.5);

hold on;

plot(t*1e9,-envelope,'r','LineWidth',1.5);

grid on;

xlabel('Time (ns)');

ylabel('Amplitude (V)');

title('Theoretical AM Envelope');

xlim([0 50]);

fprintf('\nSimulation and spectral analysis completed.\n');

Output:

Result:

The microwave signal was simulated successfully, and its frequency spectrum was analyzed using FFT and spectrum-analyzer concepts. The carrier frequency, frequency components, bandwidth, and noise characteristics were identified through frequency-domain analysis.